SOP 9.41.110 – Setup Temporary Water

Department: Project Management / Field Operations
Created by: Tom Ewing
Effective Date: [Insert Date]
Version: 1.0
Related SOPs: 9.41.035 – Conduct Site Visit; 9.41.090 – Setup Office Trailer; 9.41.095 – Setup Storage Trailer
Reference: GSL Project Manager Procedures – Mobilization

================================================================================

DOCUMENT REFERENCES

================================================================================

Training Materials (Foreman Training Binder):

- Tab 4: Staging & Job Familiarization

- Tab 22: Document Management

Job Descriptions (Policy Manual Section 7):

- 7.2.4 Foreman

- 7.2.6 Project Safety Coordinator

- 7.3.1 Operations Management

Management Directives (Policy Manual Section 9):

- 9.5 Project Management

- 9.3 Field Leadership

================================================================================

Purpose

To provide safe, reliable temporary water supply for construction use, including dust control, concrete work, cleaning, and sanitary needs.

Scope

This SOP applies to Project Managers and Field Supervisors during mobilization and throughout project execution.

Roles and Responsibilities

| Role | Responsibilities |
| --- | --- |
| Project Manager | Coordinates with owner/utility for water service connection. |
| Field Supervisor | Oversees installation of piping, hoses, and distribution. |
| Safety Coordinator | Ensures compliance with environmental and safety requirements. |

Required Documents and Tools

- Site logistics plan

- Temporary water layout plan

- Utility agreements/permits

- Safety and environmental compliance forms

Procedure

Step 1: Identify Requirements

- Determine project water needs (e.g., mixing, cleaning, sanitary).

Step 2: Coordinate with Utility/Owner

- Apply for temporary water meter or hydrant connection.

- Confirm service availability.

Step 3: Install Distribution System

- Connect to utility and install pipes/hoses.

- Provide shutoff valves and backflow prevention.

Step 4: Verify Functionality

- Test flow and pressure at distribution points.

Step 5: Safety and Environmental Review

- Ensure hoses and lines are protected from damage.

- Verify compliance with stormwater and environmental regulations.

Deliverables

| Document | Owner | Location |
| --- | --- | --- |
| Temporary Water Setup Checklist | Field Supervisor | Project Folder |
| Utility Connection Records | PM | Project Folder |
| Safety/Environmental Compliance Log | Safety Coordinator | Safety Folder |

Compliance & Review

| KPI | Target |
| --- | --- |
| Water service established before mobilization | 100% |
| Backflow prevention installed | 100% |
| Environmental compliance verified | 100% |

References

- GSL Project Manager Procedures – Mobilization

Appendix

- Temporary Water Setup Checklist

- Example Utility Connection Permit