SOP: 9.3.100 – Setup Laydown Area

Department: Project Management / Field Operations

Related SOPs:

• 9.3.010 - Setup Office Trailer

• 9.3.020 - Setup Storage Trailer

• 9.3.030 - Setup Site Fencing and Access Control

• 9.3.040 - Setup Temporary Power

• 9.3.050 - Setup Temporary Water

• 9.4.640 - Manage Storage and Logistics

Purpose

To establish a designated laydown area for material storage and staging to support efficient project operations and site safety.

Scope

This SOP applies to Project Managers and Field Supervisors during mobilization.

Roles & Responsibilities

Project Manager: Approves laydown area location per the site logistics plan.

Field Supervisor: Sets up and manages the laydown area, including organization and access.

Safety Coordinator: Reviews for compliance with safety standards and traffic flow.

Requirements

• Site logistics plan

• Laydown area setup checklist

• Material handling plan (9.2.080 – Prepare Material Handling Plan)

Procedure

Step 1 – Select Location

• Identify flat, accessible ground close to work zones.

• Ensure crane/hoist access if required for heavy materials.

Step 2 – Prepare Site

• Clear debris and grade if necessary.

• Install ground protection for heavy loads.

Step 3 – Organize Layout

• Separate storage by material type.

• Mark access lanes for forklifts and deliveries.

Step 4 – Secure Area

• Fence or barrier the laydown area as required.

• Provide lighting if area will be used after hours.

Step 5 – Safety Review

• Confirm materials are stacked properly and comply with fire and hazardous material requirements.

• Conduct weekly inspections and document findings.

Appendix

Appendix – Templates & Checklists

• Laydown Area Layout Example

• Inspection Checklist Template

Appendix – References & Training Materials

Training Materials (Foreman Training Binder):

• Tab 4: Staging & Job Familiarization

• Tab 22: Document Management

Job Descriptions (Policy Manual Section 7):

• 7.2.4 Foreman

• 7.2.6 Project Safety Coordinator

• 7.3.1 Operations Management

Management Directives (Policy Manual Section 9):

• 9.5 Project Management

• 9.3 Field Leadership